# Communication 4 — CRC Error Detection

Tutorial Group: Communication

# Goal

Extend the IR communication protocol from Communication_38kHz_Modulated_IR by adding a Cyclic Redundancy Check (CRC). The Arduino will calculate a CRC for each message and transmit it with the data. The ESP32 will calculate its own CRC from the received data and compare the result with the transmitted CRC. If the values match, the message is displayed; if they do not match, the ESP32 reports a CRC error and rejects the message.

# What You Need

Arduino Mega 2560 and USB cable

ESP32-WROOM-32 and USB cable

Standard IR LED and Vishay TSSP77038 38 kHz IR demodulator

Working Communication_38kHz_Modulated_IR system

# Why Do We Need a CRC?

A communication receiver can receive a message that looks reasonable even when one or more characters have changed. A CRC provides a way for the receiver to check whether the received data is consistent with the data that was transmitted.

CRC stands for:

Cyclic Redundancy Check

There is not one single CRC algorithm. CRCs are a family of error-detection methods. Different variants use different parameters, so both ends of a communication link must use the same variant.

The algorithm used in this tutorial is commonly called:

CRC-16/Modbus

CRC-16/Modbus is associated with the Modbus industrial communication protocol. You can search for “CRC-16/Modbus” to learn more about the algorithm and compare implementations. For this tutorial, you do not need to derive the CRC mathematics. The calculation function is provided.

## Part 1 — The CRC Calculation

Both the Arduino and ESP32 will use the same function. Copy the function exactly; changing the algorithm on one side will cause the CRC values to disagree.

uint16_t calculateCRC(const char *data) { uint16_t crc = 0xFFFF; while (*data) { crc ^= (uint8_t)*data++; for (int i = 0; i < 8; i++) { if (crc & 1) crc = (crc >> 1) ^ 0xA001; else crc >>= 1; } } return crc; }

At a high level, the function processes every character in the message and produces a 16-bit result. The important point for this tutorial is not to memorize the algorithm, but to understand that the same input should produce the same CRC.

## Part 2 — Calculate a CRC for a Known Message

Create a short test program or add temporary code to the Arduino program from Communication_38kHz_Modulated_IR. Calculate the CRC for a known message and print the result to the Serial Monitor.

const char *message = "HELLO"; uint16_t crc = calculateCRC(message);

Print the CRC in hexadecimal so it is easy to compare between the two processors.

Serial.println(crc, HEX);

The ESP32 should produce the same CRC when it calculates the CRC for the same string using the same function.

## Part 3 — Add the CRC to the Communication Protocol

Communication_38kHz_Modulated_IR transmitted the data message by itself. In this tutorial, the CRC is appended to the message.

In that tutorial, it was suggested to send the following data from the Mega 2560 to the ESP32:

v=4.87,temp=24.7

We can extend that message to include the calculated CRC at the end of the message like this:

v=4.87,temp=24.7*2564

The exact four hexadecimal characters will depend on the CRC calculated for the actual data. The asterisk is simply a delimiter that tells the receiver where the data ends and the CRC begins.

## Part 4 — Arduino: Calculate and Transmit the CRC

Modify the Arduino program so that it:

Creates the data portion of the message.

Calculates the CRC using the provided function.

Converts the CRC to four hexadecimal characters.

Appends the CRC to the message.

Transmits the complete message over the IR link.

The transmitted structure should follow this pattern:

DATA*CRC

Do not calculate the CRC over the CRC itself. The CRC is calculated from the data portion only.

## Part 5 — ESP32: Verify the CRC

Modify the ESP32 program so that it:

Receives the complete ASCII message.

Separates the data portion from the transmitted CRC.

Calculates a new CRC from the received data.

Compares the calculated CRC with the transmitted CRC.

Displays the data only when the CRC values match.

When the values match, display the voltage and temperature.

Voltage: 4.87v Temp: 24.7 °C

When they do not match, display a clear error message.

CRC ERROR

## Part 6 — Understand What the CRC Is Checking

Suppose the Arduino transmitted:

v=4.87,temp=24.7*2564

If the data is changed during transmission, the received message might instead contain:

v=2.13,temp=22.7*0E8B

The ESP32 calculates the CRC from the received data. Because the data changed, the calculated CRC should no longer match the earlier value. The ESP32 therefore reports a CRC error rather than displaying the corrupted value as if it were valid.

Notice that the corrupted value may still look completely reasonable to a person. The CRC provides an independent check of the data.

## Part 7 — Deliberately Introduce an Error

Modify the Arduino program temporarily so that the transmitted data is changed after the CRC is calculated, or otherwise create a controlled mismatch between the data and its CRC.

Confirm that the ESP32 reports CRC ERROR.

Remove the intentional error and verify that the message is accepted again.

Try more than one type of data change.

Record what you observe.

## Part 8 — Why Both Sides Must Use the Same CRC

Change one CRC parameter or calculation detail on the ESP32 and observe what happens. Restore the correct implementation when finished.

Answer:

Why does the ESP32 reject a correct message when it uses a different CRC variant?

Why is it important to specify the CRC algorithm as part of a communication protocol?

Why is “CRC-16” by itself not necessarily enough information to identify the exact algorithm?

## Part 9 — Connect CRC to the Communication Protocol

Write the complete message format used by your team. Identify which characters are data, which character separates the data from the CRC, and which characters represent the CRC.

DATA*CRC

This tutorial demonstrates CRC detection and rejection of invalid messages. A complete NACK-and-retransmission mechanism is a protocol extension you can design after completing this tutorial.

Then describe what the ESP32 does when:

The CRC matches.

The CRC does not match.

The message is incomplete.

The optical path is interrupted.

# What You Should Understand

CRC means Cyclic Redundancy Check.

CRC is an error-detection technique, not an encryption method.

CRC-16/Modbus is one specific CRC variant.

Both sides of a communication link must use the same CRC definition.

A CRC can detect many accidental changes to transmitted data.

The receiver should not treat received data as valid until its CRC check succeeds.

A message can look reasonable to a human and still fail its CRC check.

# Before Moving On

☐ I can explain what a CRC is used for.

☐ I know that CRC has multiple variants.

☐ I can identify CRC-16/Modbus as the algorithm used in this tutorial.

☐ I can explain why the same CRC calculation is required on both sides.

☐ I can explain the DATA*CRC message structure.

☐ I can demonstrate a CRC error by intentionally changing data.

☐ I can explain why the ESP32 should reject a message when the CRC does not match.

☐ I can explain how a CRC can be used to determine whether a received message contains an error.

☐ I can describe how a NACK could be used to request retransmission when a CRC check fails.